CH-8 Industrial Hyigiene Air Sampling_Homework Questions

- The type of air sampling performed when an employee wears a sampling device that draws air across a filter media for sample collection, which is sent to a laboratory is:

  - Grab Sample

  - Area Sample

  - Direct Reading Sample

  - Personal Sample

- Which airborne particle size (diameter) is most likely to cause employee long term health problems?

  - 4 micrometers

  - 40 micrometers

  - 400 micrometers

  - 4 millimeters

- What is industrial hygiene?

  - Control of health hazards arising in or from the workplace

  - Recognition and evaluation of health hazards arising from the workplace

  - Recognition and evaluation of health hazards arising in or from the workplace

  - Anticipation, recognition, evaluation, and control of health hazards arising in or from the workplace.

- In performing personal air sampling for welding fumes, where should the filter cassette be placed?

  - Attached to lapel

  - Inside welding helmet

  - On shoulder

  - At waist

- When investigating an indoor air quality issue, the first step is to:

  - Take quantitative air samples

  - Inspect the building HVAC system

  - Meet with affected employees and management

  - Survey the area for sources of air contaminants

- A charcoal tube sample was taken by drawing 40 L of air through the tube which was then analyzed for benzene. If 0.5 mg of benzene was found on the tube, what was the concentration of benzene in the air sample assuming STP? (MW of benzene is 78)

  - 3.92 ppm

  - 4.10 ppm

  - 3.50 ppm

  - 4.05 ppm

- Instantaneous or grab air samples are used to determine:

  - Composition of the air at a specific point in time and location

  - Average composition of the air during a given time period

  - Number of samples to be taken over an 8 hour shift

  - Compliance with a PEL

- In conducting a safety and health audit at a plant owned by your company, you find a solvent of potential hazard with a high probability of injury being used. What is your first course of action?

  - Stop the operation

  - Recommend the solvent be changed

  - Take air samples for analysis

  - Initiate actions to effect engineering controls